Project title

Intelligent Cooling System

Course name & code

Real – Time Analytics [CSY404]

Student name(s) : Bharath Kumar J

Student ID(s) : CU22BSC003A

Instructor’s name: Arun Sir

Abstract

The Intelligent Cooling System is a small-scale automation project designed to control the speed and activation of a cooling fan based on real-time temperature readings. The project uses an ESP32 microcontroller, a DHT11 temperature and humidity sensor, and a relay-controlled fan. To make the system more flexible and user-friendly, the final version also integrates the Blynk IoT platform, allowing remote monitoring and control through a mobile application or web dashboard.

The overall goal is to reduce unnecessary power usage while ensuring that the connected device or environment remains within safe temperature limits. The design is simple, extendable, and budget-friendly, making it ideal for students, IoT beginners, or anyone looking to automate temperature-based tasks.

Introduction

Electronic devices and modules often perform poorly or even become damaged when exposed to high temperatures. Traditionally, small cooling fans are left running continuously, even when they aren't needed. This wastes power and reduces the fan’s lifespan.

My project aims to solve this with a smarter approach: instead of running the fan all the time, the system reads the surrounding temperature and activates the fan only when conditions demand it. The ESP32 serves as the “brain,” processing sensor data and operating the relay that controls the fan.

The system was developed in three stages:

- Stage 1 — Basic Temperature Display
Sensor readings shown on the Serial Monitor.

- Stage 2 — Relay Integration
Fan automatically turns ON/OFF based on thresholds.

- Stage 3 — Full IoT Integration (Blynk)
Remote monitoring + manual/automatic control via mobile or web.

The final system behaves similar to a tiny smart air-cooling unit, with automation, remote control, and safety logic built in.

Prerequisites

Before working on this project, some familiarity with the following helps:

- Basic Arduino or ESP32 programming

- Understanding of digital pins (HIGH/LOW logic)

- Simple electronic wiring (5V/3.3V, GND, relay usage)

- Basic WiFi configuration

Hardware skills are minimal—anyone comfortable with simple circuits can replicate the setup.

Hardware Components

- ESP32 Development Board

- DHT11 Temperature & Humidity Sensor

- 5V Relay Module

- 5V or 12V Cooling Fan (depending on relay + power supply)

- Jumper wires

- USB cable for uploading code

- Optional: smartphone for Blynk application

Software / Tech Stack

| Layer | Tools Used |
| --- | --- |
| Microcontroller Programming | Arduino IDE |
| Libraries | DHT.h, WiFi.h, BlynkSimpleEsp32.h |
| IoT Platform | Blynk (mobile + web dashboard) |
| Version Control | GitHub |
| Serial Debugging | Arduino Serial Monitor |

System Workflow

- ESP32 powers on and initializes all peripherals.

- DHT11 sensor reads temperature and humidity every few seconds.

- ESP32 decides whether to switch the relay ON or OFF based on the threshold.

- In the Blynk version, the sensor data is uploaded to the app, and the fan can be controlled manually or automatically.

- The user sees real-time temperature data along with the status of the fan (ON/OFF).

The logic ensures the fan doesn’t switch too frequently, protecting both the fan and relay.

What You Accomplish With This Project

By the end of the project, the following practical skills and outcomes are achieved:

✔ Building a temperature-based automation system

You learn how to read environmental data and respond to it intelligently.

✔ Working with relays

You gain experience controlling external electrical loads safely through a microcontroller.

✔ Understanding IoT basics

You integrate cloud connectivity, remote control, and dashboards.

✔ Real debugging experience

WiFi issues, wiring mistakes, and relay logic are part of the learning curve—and you learn how to solve them.

✔ A functional, real-world mini cooling system

The final prototype can be used anywhere temperature-triggered cooling is needed.

Project Structure on GitHub

GitHub Repository:
🔗 https://github.com/Trumphblast/Intelligent-Cooling-System

The repository contains three Arduino sketches that represent the evolution of the project:

1. fan_controller-1.ino – Basic Temperature Reader

- Reads from the DHT11 sensor

- Displays temperature & humidity on Serial Monitor

- No relay or IoT involvement

2. fan_controller-2.ino – Relay Automation Added

- Controls fan using temperature thresholds

- Serial Monitor shows temperature and fan state

- Ideal for offline operation

3. fan_controller-3.ino – Blynk + WiFi + IoT Features

- Connects ESP32 to WiFi

- Sends temperature & humidity to Blynk

- Allows manual and automatic fan control

- Includes real-time dashboard feedback

This structure demonstrates a clear progression from basic sensing → automation → internet-enabled smart system.

Challenges Faced & What I Learned

While developing the project, I ran into practical issues such as:

- The ESP32 refusing to connect to a 5GHz hotspot

- Relay modules needing active-LOW logic

- DHT11 occasionally failing and requiring retries

- Blynk showing the device “offline” due to wrong WiFi or network mode

Working through these helped me understand how real IoT systems behave—far different from simple textbook examples.

Conclusion

The Intelligent Cooling System turned out to be a compact but powerful demonstration of how sensors, microcontrollers, and cloud platforms can work together to automate everyday tasks. What starts as a simple temperature reader evolves into a complete IoT solution with remote monitoring and control.

Beyond the technical features, the project reinforces problem-solving, debugging, and system-thinking skills that are essential in modern embedded development. The final system can easily be extended—such as adding speed control, using a better temperature sensor, or connecting multiple fans for more advanced cooling applications.

Overall, this project shows that even small hardware setups can become “smart” with the right logic and connectivity.